# INTERNATIONAL CONFERENCE ON HEALTH AND

# DISEASE MANAGEMENT (ICHDM 2026)

# SASTRA DEEMED TO BE UNIVERSITY,KUMBAKONAM.

# TITLE

An Intelligent EMG-Driven Knee Orthotic System for Real-Time Mobility Assistance

AUTHORS

RAGHURAM K R, VENKATESAN R, SUNDAR RAJ A, USHADEVI C

# SUPERVISOR

# VENKATESAN R

# ASSOCIATE PROFESSOR

# DEPARTMENT OF BIOMEDICAL ENGINEERING

# BATCH NO: T953

# Mode: Offline

# Venue: IEC Hall @ SASTRA UNIVERSITY

# Session & Time: II & 12.15  AM-1.15 PM

An Intelligent EMG-Driven Knee Orthotic System for Real-Time Mobility Assistance

Raghuram K R1*, Venkatesan R1, Sundar Raj A1, Ushadevi C1

1 Department of Biomedical Engineering, E. G. S. Pillay Engineering College, Nagapattinam, Tamil Nadu, India – 611002

Corresponding author email id: raghuram.r2004@gmail.com

Abstract

With the rapid growth of the global elderly population, the need for smart assistive technologies that enhance mobility and independence has become increasingly important. Older adults commonly experience joint stiffness, muscle weakness, and reduced motor control, particularly in the lower limbs, which significantly affects daily activities such as walking, standing, and climbing stairs. This project addresses this challenge through the design and development of a smart knee orthotic system that utilizes electromyography (EMG) signals to enable controlled and natural knee movement.

The proposed system integrates surface EMG electrodes, signal conditioning circuits, a microcontroller-based control unit, and an actuation mechanism consisting of a DC motor and relay circuit. Non-invasive surface electrodes acquire EMG signals from the user’s thigh muscles, ensuring comfort and suitability for real-world applications. Since raw EMG signals are weak and prone to noise, they are first amplified and filtered using a signal conditioning circuit. The processed signals, representing muscle activation levels, are interpreted by the microcontroller, which generates appropriate motor control commands. A relay circuit then drives the DC motor to actuate a mechanical mechanism that assists or replicates knee flexion and extension.

Overall, this work represents a significant step toward affordable, intelligent orthotic systems that combine biomechanical support, embedded control, and bio signal processing to address critical healthcare needs and improve the quality of life for the elderly.

Keywords

Electromyography (EMG), Smart Knee Orthosis, Assistive Technology, Surface EMG, Signal Conditioning, Embedded Control Systems, Rehabilitation Robotics

TRANSACTION DETAILS :

PRESENTATION @ ICHDM 2026